PRE BOARD EXAMINATION AUGUST 2022

Bachelor Level/First Year/I Semester          	                     Full Marks: 60

Computer Science and Information Technology                 Pass Marks: 30

Subject: Digital Logic (CSC111)                                        Time: 3 Hours

Examination Date: 22st August, 2022

Candidates are required to give their answers in their own words as for as practicable.

The figures in the margin indicate full marks.

SET A

Group ‘A’

Long Answer Question Section

Attempt any TWO questions.                                                           2×10 = 20

- 1 Implement the following function F=Ʃ(0,1,3,4,7) using

a) Decoder

b) Multiplexer

c) PLA

- Differentiate between synchronous and asynchronous sequential circuit. Design a counter as shown in the state diagram below.

- Explain different types of shift registers with necessary diagrams.

Group B

(Short Answer Question Section)

Attempt any EIGHT questions.					 (8x5=40)

- Convert (2AC5)16 to decimal, octal and binary.

- What do you mean by encoder? Design 8 to 3 encoder.

- Design a combinational circuit that multiplies 2-bit numbers, a1a0 and b1b0 to produce a 4-bit product, c3c2c1c0. Use AND gates and half-adders.

- Design a circuit which produces 2’s complement of the given four bit binary digit.

- Implement a full adder using a decoder with a truth table and logic diagram.

- Design a circuit that produces the square of a three bit number using ROM?

- Use K-map to simplify the given function in POS. Implement the simplified function using 2-input NOR- NOR gate only.

And with don’t care conditions

- Discuss race condition in J-K Flip flop and methods to overcome it.

- Write short notes on:

  - Error-detection code

  - Universal Gates

* THE END*