Kathmandu BernHardt College

Bafal, Kathmandu

Pre Board Examination-2079

Set ‘Kathmandu’

Institution: Science						 	FM: 60

Subject: Digital Logic						PM: 24

Level: B.Sc. CSIT 1st Sem						Time: 3hrs

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Candidates are required to answer the questions in their own words as far as possible.

Group A (Long Answer Question Section)

Attempt any TWO questions. 					(2×10 = 20)

- What do you mean by full adder and full sub tractor? Design a 3 to 8 line decoder using two 2 to 4 line decoder and explain it.

- Implement the following function F = Σ (1, 3, 4, 8, 14, 15) using

a. Decoder

b. Multiplexer

c. PLA

- Explain different types of shift registers with necessary diagrams.

Group B (Short Answer Question Section)		(8×5=40)

Attempt any EIGHT questions.

- What do you mean by Duality theorem? Briefly explain about De-Morgan's theorem and verify it.

- What do you mean by universal gate? Realize the following logic gates using NOR gares:

a. OR gate 	b. AND gate

- Design a circuit that produces the square of 3 bit numbers using ROM.

- Reduce the following using k-map.

F = wxy + yz + xy'z + x'y

- Implement the Full sub tractor using DE multiplexer.

- Explain Master Slave JK Flip-Flop. How it solves the problem of JK Flip-Flop.

- Differentiate between synchronous and asynchronous counter. Design mod-4 synchronous counter

- Solve the following:

F (A, B, C, D) = Σ (1, 3, 7, 11, 15) with don't care conditions as

D (A, B, C, D) = Σ (0, 2, 5)

Reduce it using k-map and draw the suitable logic diagram and realize it using NOR gate.

- Write short notes on:

a. Digital Systems

b. Gray code

c. Priority encoder

Kathmandu BernHardt College

Bafal, Kathmandu

Pre Board Examination-2079

Set ‘Kathmandu’

Institution: Science						 	FM: 60

Subject: Digital Logic						PM: 24

Level: B.Sc. CSIT 1st Sem						Time: 3hrs

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Candidates are required to answer the questions in their own words as far as possible.

Group A (Long Answer Question Section)

Attempt any TWO questions. 					(2×10 = 20)

- What do you mean by full adder and full sub tractor? Design a 3 to 8 line decoder using two 2 to 4 line decoder and explain it.

- Implement the following function F = Σ (1, 3, 4, 8, 14, 15) using

a. Decoder

b. Multiplexer

c. PLA

- Explain different types of shift registers with necessary diagrams.

Group B (Short Answer Question Section)		(8×5=40)

Attempt any EIGHT questions.

- What do you mean by Duality theorem? Briefly explain about De-Morgan's theorem and verify it.

- What do you mean by universal gate? Realize the following logic gates using NOR gares:

a. OR gate 	b. AND gate

- Design a circuit that produces the square of 3 bit numbers using ROM.

- Reduce the following using k-map.

F = wxy + yz + xy'z + x'y

- Implement the Full sub tractor using DE multiplexer.

- Explain Master Slave JK Flip-Flop. How it solves the problem of JK Flip-Flop.

- Differentiate between synchronous and asynchronous counter. Design mod-4 synchronous counter

- Solve the following:

F (A, B, C, D) = Σ (1, 3, 7, 11, 15) with don't care conditions as

D (A, B, C, D) = Σ (0, 2, 5)

Reduce it using k-map and draw the suitable logic diagram and realize it using NOR gate.

- Write short notes on:

a. Digital Systems

b. Gray code

c. Priority encoder

Kathmandu BernHardt College

Bafal, Kathmandu

Pre Board Examination-2078

Set ‘Pokhara’

Institution: Science						 	FM: 60

Subject: Digital Logic						PM: 24

Level: B.Sc. CSIT 1st Sem						Time: 3hrs

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Candidates are required to answer the questions in their own words as far as possible.

Group A (Long Answer Question Section)

Attempt any TWO questions. 					(2×10 = 20)

- Implement full adder using

a. Decoder	b. Multiplexer	c. PLA

- Discuss race condition in JK flip flop and methods to overcome it. Explain Master Slave JK Flip-Flop with logic diagram and working principle. How it solves the problem of JK Flip-Flop?

- What are the differences between combinational and sequential circuit? Design a full subtractor circuit using NOR gates.

Group B (Short Answer Question Section)		(8×5=40)

Attempt any EIGHT questions.

- Convert (2AC5)16 to decimal, octal and binary.

- Express the function F = A + B'C in sum of minterms. Reduce the following using k-map.

F = B'D + A'BC' + AB'C + ABC'

- Design a circuit that produces the cube of 2 bit numbers using ROM.

- If A = 31 and B = 17, represent them in Binary and perform

a. A – B

b. B – A using 2' complement.

- Design a 16*1 multiplexer using 4*1 multiplexers and explain it.

- With the clear circuit and timing diagram, explain the operation of Serial In Serial Out shift register when data = 1101.

- Define asynchronous counter. Design mod-7 down synchronous counter using T flip Flop.

- Solve the following:

F (A, B, C, D) = Σ (0, 5, 8, 14, 15) with don't care conditions as

D (A, B, C, D) = Σ (7, 10, 11)

Reduce it using k-map and draw the suitable logic diagram and realize it using NAND gate.

- Write short notes on:

a. De-Morgan's Law

b. Decoder

c. RS Flip-Flop

Kathmandu BernHardt College

Bafal, Kathmandu

Pre Board Examination-2078

Set ‘Pokhara’

Institution: Science						 	FM: 60

Subject: Digital Logic						PM: 24

Level: B.Sc. CSIT 1st Sem						Time: 3hrs

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Candidates are required to answer the questions in their own words as far as possible.

Group A (Long Answer Question Section)

Attempt any TWO questions. 					(2×10 = 20)

- Implement full adder using

a. Decoder	b. Multiplexer	c. PLA

- Discuss race condition in JK flip flop and methods to overcome it. Explain Master Slave JK Flip-Flop with logic diagram and working principle. How it solves the problem of JK Flip-Flop?

- What are the differences between combinational and sequential circuit? Design a full subtractor circuit using NOR gates.

Group B (Short Answer Question Section)		(8×5=40)

Attempt any EIGHT questions.

- Convert (2AC5)16 to decimal, octal and binary.

- Express the function F = A + B'C in sum of minterms. Reduce the following using k-map.

F = B'D + A'BC' + AB'C + ABC'

- Design a circuit that produces the cube of 2 bit numbers using ROM.

- If A = 31 and B = 17, represent them in Binary and perform

a. A – B

b. B – A using 2' complement.

- Design a 16*1 multiplexer using 4*1 multiplexers and explain it.

- With the clear circuit and timing diagram, explain the operation of Serial In Serial Out shift register when data = 1101.

- Define asynchronous counter. Design mod-7 down synchronous counter using T flip Flop.

- Solve the following:

F (A, B, C, D) = Σ (0, 5, 8, 14, 15) with don't care conditions as

D (A, B, C, D) = Σ (7, 10, 11)

Reduce it using k-map and draw the suitable logic diagram and realize it using NAND gate.

- Write short notes on:

a. De-Morgan's Law

b. Decoder

c. RS Flip-Flop