Kathmandu BernHardt College

Bafal, Kathmandu

Mid-Term Examination-2079

SET 'A'

Institution: BSC.CSIT	                                                               FM: 40

Subject: Digital Logic	                                                               PM: 20

Level: B.Sc. CSIT 1st Sem			                                       Time: 2hrs

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Candidates are required to give their answers in their own words as far as practicable. The figures in the margin indicate full marks.

Long Question (Attempted any one Question)              (1×10=10)

- Simplify the following using k-map Y=(ABCD') + (A'B 'CD) + (ABC) +   (A'B 'C 'D') + (ACD) + (AB'C 'D) and draw' circuits using:

a. AND – OR gates

b. NAND gates only.

You can use Not Gates wherever necessary.

- If f (P,Q,R,S) = II( 0,1,4,5,11,14,15) and d (P,Q,R,S) =II( 2,3,7,8,9,13), then design circuit using minimum number of NOR gates and basic gates.

Short Question (Attempted Any Six Question)                  (6×5=30)

- If A=61 and B=32, Represent them in Binary and perform B-A. (Use 2's complement).

- Design a combinational circuit that take 3 bit input and output is generated 1’s complement of its input using NOR gate only.

- Convert the decimal number 15.6 to equivalent binary number. Differentiate between min-term and max-term.

- Explain Half adder and Full adder with suitable truth table and logic diagram.

- Convert the given SOP expression: A’BC + ABC’D + ABD’ + A’B’D’ to POS expression.

- Define digital signal. Explain Gray code with example.

- Express the Boolean Function F= A+B’C in a sum of min-terms.

GOOD LUCK

Kathmandu BernHardt College

Bafal, Kathmandu

Mid-Term Examination-2079

SET 'A'

Institution: BSC.CSIT	                                                               FM: 40

Subject: Digital Logic	                                                               PM: 20

Level: B.Sc. CSIT 1st Sem			                           Time: 2hrs

_____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Candidates are required to give their answers in their own words as far as practicable. The figures in the margin indicate full marks.

Long Question (Attempted any one Question)              (1×10=10)

- Simplify the following using k-map Y=(ABCD') + (A'B 'CD) + (ABC) +   (A'B 'C 'D') + (ACD) + (AB'C 'D) and draw' circuits using:

a. AND – OR gates

b. NAND gates only.

You can use Not Gates wherever necessary.

- If f (P,Q,R,S) = II( 0,1,4,5,11,14,15) and d (P,Q,R,S) =II( 2,3,7,8,9,13), then design circuit using minimum number of NOR gates and basic gates.

Short Question (Attempted Any Six Question)                  (6×5=30)

- If A=61 and B=32, Represent them in Binary and perform B-A. (Use 2's complement).

- Design a combinational circuit that take 3 bit input and output is generated 1’s complement of its input using NOR gate only.

- Convert the decimal number 15.6 to equivalent binary number. Differentiate between min-term and max-term.

- Explain Half adder and Full adder with suitable truth table and logic diagram.

- Convert the given SOP expression: A’BC + ABC’D + ABD’ + A’B’D’ to POS expression.

- Define digital signal. Explain Gray code with example.

- Express the Boolean Function F= A+B’C in a sum of min-terms.

GOOD LUCK

Kathmandu BernHardt College

Bafal, Kathmandu

Mid-Term Examination-2079

SET 'B'

Institution: BSC.CSIT	                                                              FM: 40

Subject: Digital Logic	                                                              PM: 20

Level: B.Sc. CSIT 1st Sem			                                      Time: 2hrs

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Candidates are required to give their answers in their own words as far as practicable. The figures in the margin indicate full marks.

Long Question (Attempted any one Question)                  (1×10=10)

- If (A, B, C, D) = ∑ (3 , 4, 7, 8, 14) and d(A, B, C, D) =∑ (1, 6, 9, 13) design a truth table for this expression and design a circuit using

  - Using Basic gates

  - Using minimum number of NAND gates.

- If F= (A ꞌ +B) (A+D) (A+B ꞌ +C) (A ꞌ +B ꞌ +C ꞌ +D ꞌ) then minimize it using K-map and design a circuit using minimum numbers of NAND gates .

Short Question (Attempted Any Six Question)               (6×5=30)

- Why NAND and NOR gates are called Universal gates? Illustrate with example.

- Simplify π (O, 4, 5, 8, 9, 11, 15) using K-Map and write its standard SOP expression.

- Describe in your own words the characteristics of an analog and a digital signal. Convert A2.64H into its octal and decimal equivalent.

- Explain Half Subtractor and Full Subtractor with suitable truth table and logic diagram.

- Design a combinational circuit with three inputs and six outputs. The output binary number should be the square of the input binary number.

- Express the Boolean Function F= A+B’C in a product of max-terms.

- Perform the following code conversions.

    - (111O) gray= (?)BCD

    - (1430)10 = (?) Excess-3

GOOD LUCK

Kathmandu BernHardt College

Bafal, Kathmandu

Mid-Term Examination-2079

SET 'B'

Institution: BSC.CSIT	                                                             FM: 40

Subject: Digital Logic	                                                              PM: 20

Level: B.Sc. CSIT 1st Sem			                          Time: 2hrs

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Candidates are required to give their answers in their own words as far as practicable. The figures in the margin indicate full marks.

Long Question (Attempted any one Question)                  (1×10=10)

- If (A, B, C, D) = ∑ (3 , 4, 7, 8, 14) and d(A, B, C, D) =∑ (1, 6, 9, 13) design a truth table for this expression and design a circuit using

  - Using Basic gates

  - Using minimum number of NAND gates.

- If F= (A ꞌ +B) (A+D) (A+B ꞌ +C) (A ꞌ +B ꞌ +C ꞌ +D ꞌ) then minimize it using K-map and design a circuit using minimum numbers of NAND gates .

Short Question (Attempted Any Six Question)               (6×5=30)

- Why NAND and NOR gates are called Universal gates? Illustrate with example.

- Simplify π (O, 4, 5, 8, 9, 11, 15) using K-Map and write its standard SOP expression.

- Describe in your own words the characteristics of an analog and a digital signal. Convert A2.64H into its octal and decimal equivalent.

- Explain Half Subtractor and Full Subtractor with suitable truth table and logic diagram.

- Design a combinational circuit with three inputs and six outputs. The output binary number should be the square of the input binary number.

- Express the Boolean Function F= A+B’C in a product of max-terms.

- Perform the following code conversions.

    - (111O) gray= (?)BCD

    - (1430)10 = (?) Excess-3

GOOD LUCK