College of Applied Business (CAB)

Sent-up Examination, July 2022

BScCSIT / First Semester / CSC111: Digital Logic / SET A

Candidates are required to give their answers in their own words as far as practicable.

Section ‘A’	Time: 20 minutes

Attempt ALL	[5 × 2=10]

- What are the advantages of digital system over analog system?

- Define positive logic and negative logic.

- State duality theorem with example.

- Define combinational circuit.

- Convert 11002 and 01012 into gray code.

College of Applied Business (CAB)

Sent-up Examination, July 2022

BScCSIT / First Semester / CSC111: Digital Logic / SET A

Candidates are required to give their answers in their own words as far as practicable.

Section ‘B’	Time: 30 minutes

Attempt ALL	[2×5=10]

- Perform the arithmetic operation (+42)+(-13) and (-42)-(-13) in binary using the signed -2's-complement representation for negative numbers.

- Design full subtractor using NAND gates.

College of Applied Business (CAB)

Sent-up Examination, July 2022

BScCSIT / First Semester / CSC111: Digital Logic / SET A

Candidates are required to give their answers in their own words as far as practicable.

Section ‘C’	Time: 70 minutes

Attempt ALL	[5×4=20]

- Perform following arithmetic operation using r’s compliment (757 -775)8 and (CBF -EAD)16

- Use K-map to simplify the given function in POS. Implement the simplified function using 2-input NOR gate only.

F = ∏M(0,1,2,9,10,11,14)

With don’t care condition

D = ∏M(7,8,12)

- What is Parity bit? Design an odd parity generator for 3 bit message with suitable truth table and diagram.

- Explain the characteristics of IC logic families.

- Design a combinational circuit of 4-bit inputs such that it changes odd 4-input numbers into Excess-3 code and even 4-bit input numbers into gray code.