ORCHID INTERNATIONAL COLLEGE

Full Marks: 60

Time:  3 Hrs

SET A

Full Marks: 60

Time:  3 hour

Bijayachowk, Gaushala-9, Kathmandu

Pre Board Examination – 2079

Bachelor Level / First Year/ First Semester/ Science

Computer Science and Information Technology (CSC.111)

(Digital Logic)

Candidates are required to give their answer in their own words as far as practicable.

Group "A"

Attempt Any Two Questions:					       				  [2×10 = 20]

- Differentiate between synchronous and asynchronous counter. Explain the working principle of asynchronous 4-bit down counter.

- Implement the following function F= Σ (1, 2 ,8, 12, 13) using

  - Decoder

  - Multiplexer

  - PLA

- Design the sequential circuit with respect to the following state diagram using J-K flip flops.

Group "B"

Attempt Any Eight Questions: 				          					[8×5 = 40]

- Carry out the following task

  - Perform 1’s complement subtraction 110101 – 100101

  - Represent decimal number 0.125 into its binary form, octal form and hexadecimal form

- Design a combinational circuit with three inputs and one output. The output is 1 when the binary value of the inputs is an odd number.

- Express the Boolean function

  - F = x + yz as product of max-terms.

  - F = A + B’ C in a sum of min terms

- Derive the Boolean expression for sum and carry of full subtractor. Draw its combinational circuit. Implement it using only NAND gates.

- Minimize the Boolean function using K-map

F(A, B, C, D) = Σ(0, 1, 3, 5, 7, 8, 9, 11, 13, 15)

- Explain magnitude comparator.

- What is the difference between a serial and parallel transfer? Explain how to convert serial data to parallel and parallel data to serial. What type of register is needed?

- Explain JK flip flop with necessary logic diagram and truth table.

- Write Short notes on:

  - State Reduction

  - Integrated Circuit

  - ASCII Code

SET B

ORCHID INTERNATIONAL COLLEGE

Full Marks: 60

Time:  3 Hrs

Full Marks: 60

Time:  3 hour

Bijayachowk, Gaushala-9, Kathmandu

Pre Board Examination – 2079

Bachelor Level / First Year/ First Semester/ Science

Computer Science and Information Technology (CSC.111)

(Digital Logic)

Candidates are required to give their answer in their own words as far as practicable.

Group "A"

Attempt Any Two Questions:					       				  [2×10 = 20]

- Differentiate between asynchronous and synchronous circuit. Explain the working principle of decade counter.

- Implement the following function F= Σ (1, 2 ,3 ,4, 8) using

  - Decoder

  - Multiplexer

  - PLA

- Design clocked sequential circuit of the following state diagram by using JK flip-flop.

Group "B"

Attempt Any Eight Questions: 				          					[8×5 = 40]

- Carry out the following task

  - Perform 1’s complement subtraction 110101 – 100000

  - Represent decimal number 1.125 into its binary form, octal form and hexadecimal form

- Design 4-bit even parity generator.

- Derive the Boolean expression for sum and carry of full adder. Draw its combinational circuit. Implement it using only NAND gates.

- Design a 5 x 32 decoder with four 3 x 8 decoder with enable and one 2 x 4 decoder. Use block diagrams only. Explain its operation.

- Minimize the Boolean function Boolean function using K-map

F(A, B, C, D) = Σ(0, 1, 3, 5, 7, 8, 9, 10, 12, 13, 15)

- Explain Decimal Adder.

- Define register. Explain different types of shift register.

- Explain D flip flop with necessary logic diagram and truth table.

- Write Short notes on:

  - CMOS

  - Triggering Methods

  - EBCDIC