PRE BOARD EXAMINATION 2019 MARCH

Bachelor Level/First Year/I Semester          	                          Full Marks: 60

Computer Science and Information Technology         	     	Pass Marks: 24

Digital Logic                                                                                   Time: 3 Hours

Examination Date: 27th, Mar, 2019

Candidates are required to give their answers in their own words as for as practicable.

The figures in the margin indicate full marks.

SET A

Group "A"

(Long Answer Question Section)

Attempt any TWO questions.                                                         (2x10=20)

- Implement the following function F=Ʃ(0,1,3,4,7) using

a) Decoder

b) Multiplexer

c) PLA

- Differentiate between synchronous and asynchronous sequential circuit. Design a counter as shown in the state diagram below

111

001

- Explain different types of shift registers with necessary diagrams.

Group "B"

(Short Answer Question Section)

Attempt any EIGHT questions.                                                          (8x5 = 40)

- Express the complement of the following function in sum of minterms.

F(A,B,C,D) = ∑(0,2,6,11,13,14)

- Reduce the following function using k-map

F = wxy + yz + xy’z + x’y

- Design a combinational circuit with three inputs and six outputs. The output binary number should be square of the input binary number.

- Design a 5x32 decoder with four 3x8 decoder with enable and one 2x4. Use block diagrams only.

- Perform the arithmetic operations (+42) + (-12) and (-42)-12 in binary using the signed 2’s complement representation for negative numbers.

- Convert (2AC6)16 to decimal, octal and binary.

- Explain master slave J-K flip flop.

- Design full subtractor.

- Write short notes on (Any two)

  - T flip flop

  - PAL

  - TTL