PRE BOARD EXAMINATION 2019 MARCH

Bachelor Level/First Year/I Semester          	                          Full Marks: 60

Computer Science and Information Technology         	     	Pass Marks: 24

Digital Logic                                                                                   Time: 3 Hours

Examination Date: 27th, Mar, 2019

Candidates are required to give their answers in their own words as for as practicable.

The figures in the margin indicate full marks.

SET B

Group A (Long Answer Question Section)

Attempt any TWO questions.                                                     (2x10=20)

- Implement the following function F=Ʃ(0,1,2,4,7) using

a) Decoder

b) Multiplexer

c) PLA

- Differentiate between synchronous and asynchronous sequential circuit. Design a counter as shown in the state diagram below

- Explain working principle of binary up/down counter with necessary

diagram.

Group B

(Short Answer Question Section)

Attempt any EIGHT questions.                                                         (8x5 = 40)

- Use K-map to simplify the given function in POS. Implement the simplified function using 2-input NOR gate only.

F = ∏M(0,1,2,9,10,11,14)

With don’t care condition

D = ∏M(7,8,12)

- Discuss race condition in J-K Flip flop and methods to overcome it.

- Convert (2AC5)16 to decimal, octal and binary.

- Design a circuit that produces the square of three bit number using ROM?

- Design a 2-bit magnitude comparator.

- Design a BCD to excess -3 code converter.

- Perform the arithmetic operations (+42) + (-13) and (-42)-13 in binary using the signed 2’s complement representation for negative numbers.

- Show that a positive logic NAND gate is a negative logic NOR gate and vice versa.

- Write short notes on (Any two)

  - CMOS

  - Don’t care condition

  - Half subtractor