Asian School of Management and Technology

SET A

Full Marks: 60

Time: 3 Hrs

(Affiliated to Tribhuvan University)

Gongabu, Kathmandu

B.Sc. CSIT / First Semester / CSC 111: Digital Logic

Pre Board Examination 2075

Candidates are required to answer the question in their own words as far as practicable.

Long Answer Questions:

Attempt any TWO Questions:						2×10=20

- What do you mean by BCD addition? Design a logic circuit for BCD addition and explain it working principle with its truth table and logic diagram.

- What do you mean by PLA? A combinational circuit is defined by the functions:

F1 (A, B, C) =∑ (1, 2, 3, 6, 7)

F2 (A, B, C) =∑ (0, 2, 4, 5, 7). Implement the circuit with PLA.

- Design a BCD ripple counter with its state diagram, timing diagram, and logic diagram and explain the working principle.

Short Answer Questions:

Attempt any EIGHT Questions:					8×5=40

- What do you mean by ROM? Implement F(A,B,C,D)= ∑(0,1,3,4,8,14,15) using ROM and MUX.

- What do you mean by code converter? Design 3 bit gray code to binary code converter.

- Design a MOD-6 UP/DOWN synchronous counter with state diagram, truth table and logic diagram.

- Prove that:

  - WX’Y+X+XZ’+WXZ’+W’Y=X+Y

  - W’Y+W’X+WX’Y+XY=Y+W’X

- Simplified the Boolean function F(A,B,C,D)= ∑ (0,3,6,7,11,12,15) by using Tabulation method.

- What do you mean by shift register? Discuss about 3-bit PISO shift register with logic circuit and timing diagram.

- “J-K flip –flop is refinement of S-R flip-flop”. Illustrate this statement.

- Perform the following operation:

  - (11000110)2-(10111011)2 using 1’s complement.

  - (726)10-(856)10 using 10’s complement.

  - (165)10-(102)10 using BCD subtraction.

- Write short note on(Any TWO):

  - Triggering of flip flop.

  - Digital logic families.

  - Integrated circuits.

***Good Luck***

Asian School of Management and Technology

SET B

Full Marks: 60

Time: 3 Hrs

(Affiliated to Tribhuvan University)

Gongabu, Kathmandu

B.Sc. CSIT / First Semester / CSC 111: Digital Logic

Pre Board Examination 2075

Candidates are required to answer the question in their own words as far as practicable.

GROUP-A

Long Answer Questions:

Attempt any TWO Questions:						2×10=20

- What do you mean by magnitude comparator? Design a logic circuit for 4 bit magnitude comparator and explain it.

- Design a BCD ripple counter with its state diagram, timing diagram, and logic diagram and explain the working principle.

- Design a sequential circuit with JK flip flops to satisfied the following state equations:

A (t+1) =A’B’CD+A’B’C+ACD+AC’D’

B (t+1) =A’C+CD’+A’BC’

C (t+1) =B

D (t+1) =D’

GROUP-B

Short Answer Questions:

Attempt any EIGHT Questions:					8×5=40

- What do you mean by multiplexer? Implement F(A,B,C,D)= ∑(0,1,3,4,8,14,15) using multiplexer.

- Perform the following operation:

  - (11000110)2-(10111011)2 using 2’s complement.

  - (726)10-(856)10 using 9’s complement.

  - (65)10-(12)10 using BCD subtraction.

- What do you mean by code converter? Design 3 bit binary to gray code converter.

- Design a MOD-5 UP/DOWN synchronous counter with state diagram, truth table and logic diagram.

- Described the clocked master-slave S-R flip-flops with its operation table

- Prove that:

  - AB’C+B+BD’+ABD’+A’C=B+C

  - A’C+A’B+AB’C+BC=C+A’B

- Construct full adder by using decoder.

- What do you mean by shift register? Design 4 bit PISO shift register with logic circuit and timing diagram.

- Write short note on(Any TWO):

  - Alphanumeric code

  - PLA

  - Decoder

***Good Luck***