NAGARJUNA COLLEGE OF INFORMATION TECHNOLOGY

Full Marks: 60

Pass Marks: 30

Time: 3 Hrs

Hariharbhawan, Pulchowk

Lalitpur

Pre- Board Exam -2075

BSc. CSIT / First Semester / CSC 111: Digital Logic

Candidates are required to their answers in their own words as far as practicable

Group ''A''

Attempt any TWO questions:                                                                                                   [2×10=20]

- Implement Boolean function  F=(1,2,5,7) using

- Decoder

- MUX

- PLA

- Differentiate between PAL and PLA. Also design MOD-12 counter using D- flip flop.

- Design 4-bit magnitude comparator. Also explain design steps for combinational circuit

Group ''B''

Attempt any EIGHT questions:                                                                                           [8×5=40]

- If A = (35)10 and B = (95)10 , then calculate  (A-B)  and (-A-B)  using 2’s  and 1’s complement method

- Define flip flop. Also explain  S-R  flip flop with its logical diagram , characteristic table, excitation table and wave form

- You are provided with a bit sequence of 1001 to operate with register in which data are entered simultaneously and retrieved one at a time. Describe the store and retrieve process with supportive diagram and timing diagram.

- Plot the given Boolean function in truth table and reduce it using K-map.

F= ABC’ + B’C+ A’B’C+ B’

- Design 5*32 decoder using 2*4 decoder

- Differentiate between Mealy and Moore sequential machine model with help of diagram

- What is digital system? Write down advantages and disadvantages of digital system

- Design a combinational circuit with three inputs (X, Y, Z) and three outputs (A, B, C).

- When the triple (X,Y,Z) is taken as the 3-bit representation of a binary number, then (0,0,0) corresponds to 0, (0,0,1) to 1, (0,1,1) to 3, (1,0,1) to 5 and so on.

- Hence, X is the most significant bit while Z is the least significant bit.

- Similarly for the triple (A,B,C). When the binary input (X, Y, Z) is 0, 1, 2 or 3, the binary output is one greater than the input.

- When the binary input is 4, 5, 6 or 7, the binary output is one less than the input.

- Write the Truth Table for the inputs and outputs.

- Simplify the resulting expressions for A, B and C outputs (use algebra or maps).

- Define adder? design combined adder and subtractor