PRE –BOARD EXAMINATION-2075

Digital Logic (SET A)

Time: 3hr Full Marks: 60 / Pass Marks: 30

Long answer questions:

Attempt any two questions:					(2*10=20)

- What is magnitude comparator?Design and Explain 4 bit magnitude comparator.

- What are asynchronous counter? Design synchronous counter that counts the sequences of 1-3-6-7-1-0 and repeat using T flip-flop.

- Implement the following functions using a. Multiplexer b. Decoder c. PAL

F(A,B,C,D)=∑(0,1,5,7,8,10,14,15)

Short answer questions:

Attempt any eight questions:						(8*5=40)

- If A=-29 and B=-20 then perform A-B and B-A using 2’s complement.

- If F(P,Q,R,S)= Σ (3,4,7,8,14) and d(P,Q,R,S)= Σ(1,6,9,13),Simplify it using K-map and  design circuit using minimum number of  NAND gates.

- Design 8 to 3 line Encoder using NAND  gate only.

- Design a full subtractorcircuit usingProgrammable Logic Array.

- Design and Explain4 bitSerial in serial Out shift Register with timing Diagram.

- Implement a circuit which accepts 3 bit inputs and output exceed square of input by3using ROM.

- Design asynchronous Decade counter with state diagram, logical diagram and timing diagram.

- Explain about flipflop which is the single input version of RS flip-flop with logical diagram,characteristicstable,characteristics equation and excitation table.

- Write short notes on:

a.) State Diagram

b.) Reflected Code

c.) Johnson counter

-----BEST OF LUCK----

PRE-BOARD EXAMINATION-2075

Digital Logic (SET B)

Time: 3hr    Full Marks: 60 / Pass Marks: 30

Long Answer Questions:

Attempt any two questions.					(2x10=20)

- What are asynchronous counter? Design synchronous Decade counter using T Flip-flop.

- Design and Explain Decimal adder.

- Implement the following functions using a. Multiplexer b. Decoder c. PAL

F(A,B,C,D)=∑(0,1,2,4,7,8,10,14)

Short Answer Questions:

Attempt any eight questions.				(8x5=40)

- If A=-19 and B=-20 then perform A-B and B-A using 2’s complement

- If F(P,Q,R,S)=𝜋(0,1,4,5,11,14,15) and d(P,Q,R,S)= 𝜋(2,3,7,8,9,13),Simplify it using K- Map and  design circuit using minimum number of NAND gates.

- Design 1x8 DEMUX using NOR gate only.

- Design a full adder circuit using Programmable  Logic Array.

- Implement a circuit which accepts 3 bit binary input and output exceed five times the input by 2 using ROM.

- Design and Explain  3-bit Parallel In Serial Out Shift Register along with Timing diagram.

- Design asynchronous MOD-12 counter with state diagram, logical diagram and timing diagram.

- Explain about the flipflop which is  single input version of JK flip-flop with logical diagram, characteristics table, characteristics equation and excitation table.

- Write short notes on (any two):

  - Transition Table

  - Ring counter

  - Error Detection Codes

----BEST OF LUCK----