PRE-BOARD EXAMINATION CHAITRA 2080

B.Sc.CSIT 1st Semester				Full Marks: 60

Subject: Digital Logic   				Pass Marks: 30

Examination Date: 9th Jestha, 2080        		Time: 3 Hours

Candidates are required to give their answers in their own words as practicable.

The figures in the margin indicate full marks.

SET A

Attempt any TWO questions. (2x10 = 20)

- Implement the following function using PLA and PAL.

a) F (W X Y Z) = Σ ( 0, 1, 3, 4, 8, 9, 10)

b) F (W X Y Z) = Σ ( 1, 3, 5, 6, 11, 13, 14)

- Explain working principle of 4-bit binary ripple counter along

- with logic diagram and timing diagram.

- Write short notes on.

i) Edge triggering vs level triggering

ii) T-flip flop

iii) Edge-triggered flip flop

iv) Direct Inputs

Attempt any EIGHT questions. (8x5 = 40)

- Convert (2BC5)16 to decimal, octal and binary.

- Design a 3 to 8 line decoder using two 2 to 4 line decoders and explain it.

- Implement the following Boolean function with an

8x1 multiplexer.

F(A,B,C,D) = Σ (0,3,5,6,8,9,14,15)

- Design an encoder using universal gates.

- Design a combinational circuit that converts a decimal digit from the 2 4 2 1 code to 8 4 -2 -1 code.

- Draw a NOR logic diagram that implements the complement of the following function.

F(A,B,C,D) = ∑(0,1,2,3,4,8,9,12)

- Draw the AND-OR gate implementation of the following function after simplifying it in

a) Sum of products

b) Product of sums

F(A,B,C,D) = ∑(0,2,5,6,11)

- Simplify the following functions and implement them with two-level NOR gate circuits.

F = wx’ + y’z’ + w’yz’

- Find the complement of F = x +yz; then show that F.F’ = 0 and F+F’ =1

PRE-BOARD EXAMINATION CHAITRA 2080

B.Sc.CSIT 1st Semester				Full Marks: 60

Subject: Digital Logic   				Pass Marks: 30

Examination Date: 9th Jestha, 2080        		Time: 3 Hours

Candidates are required to give their answers in their own words as practicable.

The figures in the margin indicate full marks.

SET A

Attempt any TWO questions. (2x10 = 20)

- Implement the following function using PLA and PAL.

a) F (W X Y Z) = Σ ( 0, 1, 3, 4, 8, 9, 10)

b) F (W X Y Z) = Σ ( 1, 3, 5, 6, 11, 13, 14)

- Explain working principle of 4-bit binary ripple counter along

- with logic diagram and timing diagram.

- Write short notes on.

i) Edge triggering vs level triggering

ii) T-flip flop

iii) Edge-triggered flip flop

iv) Direct Inputs

Attempt any EIGHT questions. (8x5 = 40)

- Convert (2BC5)16 to decimal, octal and binary.

- Design a 3 to 8 line decoder using two 2 to 4 line decoders and explain it.

- Implement the following Boolean function with an

8x1 multiplexer.

F(A,B,C,D) = Σ (0,3,5,6,8,9,14,15)

- Design an encoder using universal gates.

- Design a combinational circuit that converts a decimal digit from the 2 4 2 1 code to 8 4 -2 -1 code.

- Draw a NOR logic diagram that implements the complement of the following function.

F(A,B,C,D) = ∑(0,1,2,3,4,8,9,12)

- Draw the AND-OR gate implementation of the following function after simplifying it in

a) Sum of products

b) Product of sums

F(A,B,C,D) = ∑(0,2,5,6,11)

- Simplify the following functions and implement them with two-level NOR gate circuits.

F = wx’ + y’z’ + w’yz’

- Find the complement of F = x +yz; then show that F.F’ = 0 and F+F’ =1