Nagarjuna College of Information Technology

Full Marks: 60

Pass Marks: 24

Time: 3 hrs.

Shankhamul -09, Kathmandu

Pre-Board Examination 2080

Bachelor Level / First Year/ First Semester/ Science

Computer Science and Information Technology (CSC 111)

(Digital Logic)

Candidate are required to give their answers in their own words as far as possible.

The figures in the margin indicate full marks.

Group – A

Attempt any TWO questions:							2×10=20

- Design the mod-6 asynchronous counter and explain with truth table.

- Design a magnitude comparator using logic gates and truth table.

- Implement the following function F=∑(1,2,3,4,8) using

- Decoder

- Multiplexer

- PLA

Group – B

Attempt EIGHT questions:							8×5=40

- Differentiate between Analog and Digital system.

- Convert (82.58)10   to octal, hexadecimal and binary number.

- What is the difference between a serial and parallel transfer? Explain how to convert serial data into parallel and parallel data into serial. What type of register needed?

- Explain JK flipflop with necessary logic diagram.

- Design a half adder logic circuit using NOR gates only.

- Design 3-bit synchronous up counter.

- Design and explain the BCD adder (decimal adder) with necessary truth table and diagram

- Reduce the following expression using K-map.

F=wxy+yz+xy’z+x’y

- Differentiate between a counter and shift register.