National College of Computer Studies

(NCCS-College of IT & Management)

Final Examination (2021)

BSc.CSIT/First Semester/ CSC 111 : Digital Logic

# Candidates are required to answer the questions in their own words as far as practicable.

Full Marks: 60

Pass Marks: 30

Time: 3 Hours

Set B

Attempt any two questions.									        	 [2×10=20]

- What are asynchronous counter? Design synchronous Decade counter using T Flip-flop.

- Design and Explain Decimal adder.

- Implement the following functions using a. Multiplexer b. Decoder c. PAL

F(A,B,C,D)=∑(0,1,2,4,7,8,10,14)

Attempt any eight questions.								                [8×5=40]

- If A=-19 and B=-20 then perform A-B and B-A using 2’s complement

- If F(P,Q,R,S)=𝜋(0,1,4,5,11,14,15) and d(P,Q,R,S)= 𝜋(2,3,7,8,9,13),Simplify it using K- Map and  design circuit using minimum number of NAND gates.

- Design 1x8 DEMUX using NOR gate only.

- Design a full adder circuit using Programmable Logic Array.

- Implement a circuit which accepts 3 bit binary input and output exceed five times the input by 2 using ROM.

- Design and Explain 3-bit Parallel In Serial Out Shift Register along with Timing diagram.

- Design asynchronous MOD-12 counter with state diagram, logical diagram and timing diagram.

- Explain about the flip-flop which is single input version of JK flip-flop with logical diagram, characteristics table, characteristics equation and excitation table.

- Write short notes on (any two):

  - Transition Table

  - Ring counter

  - Error Detection Codes

********